### Random Finite Sets (RFS)

### Random Finite Sets (RFS) provide a unified mathematical framework for multi-target tracking where both the number of targets and their states are uncertain. Instead of treating each target and measurement individually, RFS models the entire collection of targets and measurements as random sets. This formulation eliminates the need for explicit data association, which is often computationally complex in traditional tracking methods. In an RFS-based approach, the Bayesian filtering framework is extended to random sets, allowing for recursive updates of the multi-target probability distribution as new sensor data arrives. The framework naturally handles target births and deaths, false alarms, and clutter, making it well-suited for dynamic and uncertain environments. Several filters, such as the Probability Hypothesis Density (PHD) and Cardinalized PHD (CPHD) filters, have been derived from this theory to provide tractable implementations.

### The RFS framework offers a powerful way to handle multi-target tracking problems with unknown and varying numbers of targets while inherently managing data association ambiguity. It provides a rigorous probabilistic foundation that unifies detection, estimation, and tracking under one model, leading to robust performance in cluttered environments. However, the main limitation lies in its computational complexity and the need for simplifying assumptions (like Poisson distributions) to make implementation feasible. These approximations can reduce estimation accuracy, especially when target dynamics or measurement models deviate from assumed distributions. Additionally, the standard RFS framework does not inherently preserve individual target identities, requiring extensions such as Labeled RFS (LRFS) models for identity tracking.

### Probability Hypothesis Density (PHD) Filter

### The Probability Hypothesis Density (PHD) filter, proposed by Ronald Mahler, is a practical implementation derived from RFS theory. It propagates the first-order statistical moment (the PHD) of the multi-target posterior distribution instead of the full joint distribution, thus reducing computational complexity. The PHD filter follows a recursive Bayesian structure with prediction and update steps. During prediction, it uses a motion model to estimate target survival and introduces potential new targets (births). In the update step, it incorporates sensor measurements while accounting for detection probability and clutter. Two common implementations are the Gaussian Mixture PHD (GM-PHD) filter, which represents the PHD as a mixture of Gaussians for linear Gaussian systems, and the Sequential Monte Carlo (SMC-PHD) filter, which uses particles for non-linear, non-Gaussian systems. These approximations allow real-time tracking of multiple targets in cluttered environments without explicit data association.

### The PHD filter significantly reduces the complexity of multi-target tracking by avoiding combinatorial data association and operating directly on the single-target state space. It efficiently handles target births, deaths, missed detections, and false alarms, offering robust performance in dense tracking scenarios. The GM-PHD filter, in particular, enables closed-form updates for Gaussian models, making it computationally efficient for practical systems. However, the standard PHD filter does not maintain target identities or track histories, limiting its use in applications requiring continuous track labeling. Furthermore, its Poisson-based assumptions can result in inaccuracies in estimating the exact number of targets (cardinality errors), especially under low detection probabilities. The CPHD and labeled RFS variants have been developed to mitigate these issues, but at the cost of increased computational demand.

### Sequential Monte Carlo (SMC) Methods / Particle Filters

### Sequential Monte Carlo (SMC) methods, or particle filters, are a class of algorithms designed to estimate the state of a dynamic system from noisy observations by representing the probability distribution of the state using a set of weighted random samples called particles. In multi-target tracking, SMC methods approximate the posterior probability density function (PDF) of each target’s state recursively over time. The process begins with an initialization step where a population of particles is generated to represent the prior belief of the target’s state. Each time a new measurement is received, the particles are propagated forward according to a motion model (prediction), reweighted based on how well they match the observed data (update), and then resampled to eliminate low-weight particles. This recursive cycle allows the filter to approximate complex, nonlinear, and non-Gaussian dynamics, making it well-suited for challenging tracking scenarios, especially when combined with frameworks like Random Finite Sets (RFS) to handle multiple targets simultaneously.

### The main advantage of SMC methods is their flexibility such as their ability to handle nonlinear motion models, non-Gaussian noise, and multimodal probability distributions that arise in ambiguous or cluttered environments. They provide a robust probabilistic framework that adapts to uncertainty and can be extended for multi-target tracking through the SMC-PHD filter. However, their computational cost increases rapidly with state dimensionality, a challenge known as the “curse of dimensionality.” Without effective resampling, particle degeneracy can occur, leading to poor estimates as most particles lose significance. Moreover, the filter’s performance depends heavily on parameters such as the number of particles, proposal distributions, and resampling strategies, which often require careful tuning. These factors make real-time implementation in complex, high-dimensional tracking problems computationally demanding.

### Transformer-Based Multi-Target Trackers

### Transformer-based multi-target tracking leverages the self-attention mechanism from deep learning to jointly perform object detection and data association within an end-to-end framework. Unlike traditional trackers that process sequences frame by frame, Transformers process all detections and track queries in parallel, allowing them to learn long-range dependencies and global spatio-temporal relationships. The network uses three key components i.e. queries, keys, and values, to compute attention scores that represent the correspondence between existing tracks and new detections. Queries encode the historical state of each tracked object, while keys and values represent the features of current detections. Transformer-based trackers such as TransTrack, TrackFormer, and DeepAF combine convolutional feature extractors with Transformer encoders and decoders to perform prediction and association simultaneously. This “joint detection and tracking” paradigm replaces traditional matching algorithms with learned attention-based associations, enabling the model to robustly track multiple objects even in crowded or cluttered scenes.

### Transformer-based trackers offer several key advantages, including the ability to process detection and association in a unified end-to-end pipeline, reducing dependency on hand-crafted data association modules such as the Hungarian algorithm. Their global attention mechanism captures contextual relationships among all detections, improving robustness against occlusions and identity switches. They also excel at maintaining consistent object identities across long sequences by leveraging temporal dependencies. However, these benefits come at the cost of high computational and memory requirements due to the quadratic complexity of the self-attention mechanism. Training Transformers can be slow to converge, and they may struggle with small or partially visible objects because their attention mechanisms tend to prioritize larger, more distinct features. Despite these challenges, ongoing research into lightweight and hierarchical attention architectures is improving the efficiency and scalability of Transformer-based trackers.

### Graph Neural Network (GNN)-Based Multi-Object Trackers

### Graph Neural Networks (GNNs) enhance multi-object tracking by modeling the relationships between objects as a graph structure, where detections are represented as nodes and their spatial or temporal relationships as edges. This approach enables context-aware reasoning across multiple objects and frames. In the tracking-by-detection paradigm, object detections from consecutive frames are connected through a spatio-temporal graph. Node features are derived from object appearance (via CNNs), while edge features encode motion and geometry. Through iterative message passing, the GNN refines node and edge embeddings to infer which detections belong to the same trajectory. More advanced, joint detection and tracking frameworks integrate GNNs directly into end-to-end neural architectures, allowing them to simultaneously detect and associate objects by leveraging learned spatial-temporal interactions. By updating node representations through their neighbors, GNN-based trackers can reason globally across the scene and handle challenging cases like occlusions or target reappearances.

### The strength of GNN-based trackers lies in their ability to explicitly model object interactions and global context, improving robustness in crowded or cluttered environments. Their message-passing mechanism allows information sharing between related detections, enhancing performance in situations involving occlusion or identity ambiguity. They can dynamically adapt as objects appear or disappear and can be trained end-to-end for optimized detection and tracking performance. However, GNN-based methods are computationally expensive, especially when the number of objects or connections grows, as the graph’s size directly affects processing time. They are also sensitive to graph construction errors e.g. poorly defined nodes or edges can lead to incorrect associations. Furthermore, over-smoothing in deeper networks can blur distinctions between similar-looking objects, and scalability becomes a concern for long video sequences. Despite these challenges, GNNs remain a promising approach for learning structured, relational representations in complex multi-object tracking scenarios.

### Transformer-Enhanced Multi-Object Tracking (MOT)

### Transformer-enhanced multi-object tracking (MOT) leverages the self-attention mechanism of Transformer architectures to unify detection and data association into a single, end-to-end framework. Unlike traditional “tracking-by-detection” methods that rely on multiple separate stages, these trackers model the temporal dependencies and global context across video frames directly. They formulate tracking as a set prediction task, where object identities are maintained through attention-based “track queries” that evolve over time. The Transformer decoder typically employs two types of queries i.e. track queries, which represent existing objects and maintain their identities across frames, and object queries, which detect newly appearing objects. During inference, attention layers match these queries with image features, allowing the network to automatically associate detections without explicit data association algorithms. Prominent architectures such as TransTrack, TrackFormer, MOTR, and Motion-Aware Transformer (MATR) demonstrate the effectiveness of this “tracking-by-attention” paradigm, improving robustness to occlusions and dynamic scenes by using global reasoning over both spatial and temporal information.

### Transformer-based MOT approaches provide significant advantages in tracking accuracy, robustness, and architectural simplicity. The self-attention mechanism enables context-aware tracking, allowing the model to reason globally across scenes and maintain identities even under occlusions or camera motion. Their end-to-end structure eliminates the need for handcrafted data association steps, leading to a cleaner and more unified design. Moreover, track queries retain temporal continuity, supporting better identity preservation across frames. However, these methods are computationally expensive, as attention operations scale quadratically with the number of elements, leading to slower inference speeds. They also require large training datasets and can exhibit slow convergence during optimization. Performance often degrades for small or overlapping objects, and query conflicts in crowded scenes can still cause identity switches. Despite these challenges, Transformers have established a powerful new direction in multi-object tracking, inspiring ongoing research into lightweight, efficient attention architectures.

### Gaussian Mixture Probability Hypothesis Density (GM-PHD) Filter

### The Gaussian Mixture Probability Hypothesis Density (GM-PHD) filter is a computationally efficient implementation of the PHD filter derived from Random Finite Set (RFS) theory. It represents the multi-target intensity function (the first-order statistical moment of the multi-target posterior) as a weighted sum of Gaussian components, each corresponding to a potential target state. The filter operates recursively through prediction and update steps: during prediction, existing Gaussian components are propagated forward based on a linear motion model (survival), while new components are introduced to account for target births. In the update step, component weights are adjusted according to how well each Gaussian matches the received measurements, considering both detection likelihood and clutter. To manage the computational load, pruning removes low-weight components, and merging combines nearby Gaussians to control model complexity. Finally, the state extraction process identifies active targets by selecting Gaussian components whose weights exceed a certain threshold. This structured, closed-form implementation makes the GM-PHD filter highly suitable for tracking multiple targets under linear-Gaussian conditions.

### The GM-PHD filter offers several advantages, particularly its computational efficiency and closed-form recursion for linear-Gaussian models. By using a Gaussian mixture to represent the intensity function, it avoids the combinatorial data association problem, making it highly effective in cluttered environments and when the number of targets varies. The Gaussian structure also provides a smooth and interpretable representation of the target density. However, the filter relies heavily on linear-Gaussian assumptions, limiting its accuracy in nonlinear or non-Gaussian scenarios. Like the standard PHD filter, it does not preserve individual target identities, only estimating the expected number and states of targets. Furthermore, the Poisson RFS assumption can lead to fluctuations in target count estimates, particularly when detection probabilities are low. The GM-PHD’s performance is also sensitive to parameters controlling pruning, merging, and thresholding, which require careful tuning. In complex, highly nonlinear tracking environments, more flexible approaches such as the Sequential Monte Carlo PHD (SMC-PHD), Cardinalized PHD (CPHD), or Labeled RFS filters are often preferred for improved robustness and identity management.

### Labelled Multi-Bernoulli (LMB) Filter

### The Labelled Multi-Bernoulli (LMB) filter is a multi-target tracking algorithm formulated within the Random Finite Set (RFS) framework. It extends earlier RFS-based methods such as the Probability Hypothesis Density (PHD) and Cardinalized PHD (CPHD) filters by introducing explicit target identities through labelled RFSs. In a standard RFS, the multi-target state is represented as an unordered set of states, which supports variable target counts but discards identity information. A labelled RFS, in contrast, represents the multi-target state as a set of states often encoded as a tuple (birth_time,birth_index) to ensure persistent target identity across time.

### Each potential target is represented by a Bernoulli component, defined by its probability of existence rrr and spatial density ppp. The recursive process consists of prediction and update steps. During prediction, existing components are propagated using motion and survival models, while new components with unique labels are introduced for births. In the update step, the filter evaluates measurement likelihoods and updates the Bernoulli components based on marginal association probabilities which is an approximation that maintains tractability compared to the full Generalized LMB (GLMB) filter. After the update, target states are extracted by selecting components whose existence probabilities exceed a threshold, using their associated spatial densities for state estimation.

### The LMB filter provides a principled solution to one of the main shortcomings of earlier RFS filters (the loss of target identity) by explicitly maintaining labelled trajectories. This ensures track continuity and supports long-term identity preservation across occlusions and clutter. It also offers accurate cardinality estimation and improved computational efficiency by avoiding the exponential hypothesis growth that characterizes the GLMB filter. Moreover, the LMB inherits the robustness of RFS methods to false alarms, missed detections, and clutter. However, its efficiency stems from an independence assumption between targets, which may degrade performance in highly interactive or dense scenes. While it closely approximates the GLMB filter, it can exhibit slightly lower accuracy in scenarios requiring detailed reasoning over explicit data associations. Additionally, approximation errors introduced during marginal association can affect estimation precision, and careful tuning of parameters for pruning and merging is required. Despite these trade-offs, the LMB filter achieves an excellent balance between computational tractability and identity-aware multi-target tracking, making it a cornerstone in modern RFS-based tracking research.

### Integrated Probabilistic Data Association (PDA) with Machine Learning Feedback

### Integrated Probabilistic Data Association (PDA) with Machine Learning Feedback (MLF) represents a hybrid approach that combines probabilistic filtering with data-driven learning to enhance multi-target tracking under uncertainty. Traditional PDA and Joint PDA (JPDA) filters compute association probabilities between tracks and measurements based on predefined metrics such as the Mahalanobis distance which is derived from kinematic models. Machine learning feedback enriches this process by introducing learned cues and adaptive models. Deep learning architectures, including convolutional neural networks (CNNs) and recurrent networks (e.g., LSTMs), can learn affinity metrics that capture appearance, motion patterns, or inter-object relationships beyond handcrafted features. ML components can also model adaptive noise and uncertainty, predicting covariance values for non-stationary environments, and directly refine association probabilities by weighting or modifying the PDA filter’s probabilistic outputs. Specialized modules, such as Re-identification (ReID) networks, assist in track recovery after occlusions, while clutter classifiers can suppress false alarms before filtering. A notable implementation is the Integrated PDA with Machine Learning Feedback (IPDA-MLF) for distributed passive tracking, where local IPDA filters fuse sensor data and a CNN provides feedback on low-observability conditions, improving association reliability and overall state estimation through centralized fusion.

### The fusion of probabilistic data association with ML feedback provides several synergistic benefits. It combines the statistical rigor and uncertainty handling of classical filters with the pattern recognition capabilities of deep learning, leading to enhanced robustness against clutter, occlusion, and measurement noise. By learning rich feature representations, ML-assisted PDA systems achieve higher association accuracy and improved state estimation in complex environments where handcrafted models fall short. ML modules can also reduce filter workload by predicting measurement ambiguity, thus lowering computational complexity. However, this hybrid paradigm faces notable challenges. The ML component’s performance depends heavily on the availability and quality of labeled training data, which can be costly to obtain for all operational scenarios. Integrating probabilistic and neural components requires careful fusion strategies to reconcile statistical and learned uncertainties. Moreover, ML models may suffer from domain generalization issues, performing poorly when applied to unseen conditions or sensor modalities. Finally, the addition of deep learning components increases system complexity and computational requirements. Despite these challenges, ML-enhanced PDA frameworks like IPDA-MLF represent a promising direction for adaptive, data-driven multi-target tracking that unifies model-based reasoning with learning-based perception.

### Local Sensor Integrated Probabilistic Data Association (LS-IPDA) Filter

### The Local Sensor Integrated Probabilistic Data Association (LS-IPDA) filter is a distributed multi-target tracking approach designed for multi-sensor networks. Instead of relying on a centralized fusion system to process all sensor data, each sensor node runs its own local IPDA filter on its respective measurements. This decentralization reduces computational overhead and communication bandwidth, making the framework scalable to large sensor deployments. A key feature of LS-IPDA is the use of the Probability of Target Existence (PTE) as a metric to manage track quality at each local node. Each sensor independently initiates, maintains, and terminates tracks based on the PTE, effectively rejecting false alarms early in the process. For sensors with limited observability such as bearing-only or amplitude-only sensors, the LS-IPDA can perform pseudo-tracking, which estimates measurable quantities (e.g., angle, rate of change) even when full kinematic states cannot be determined. After local processing, refined track information (state estimates, covariances, and existence probabilities) is transmitted to a central fusion center, where these local tracks are combined to form a coherent global picture. The central fusion module accounts for asynchronous or out-of-sequence measurements and synthesizes multi-sensor information for robust target state estimation.

### The LS-IPDA framework provides several critical advantages for large-scale, distributed sensing environments. By performing local processing, it dramatically reduces communication load and only compact track summaries are sent to the fusion center instead of raw data. This architecture is therefore highly scalable, suitable for networks with constrained bandwidth and numerous sensors. Its local filtering stage also provides early clutter rejection, improving computational efficiency and overall system robustness. The pseudo-tracking capability further enhances adaptability, enabling the use of heterogeneous sensor types with different levels of observability. However, these benefits come with challenges. Some implementations simplify sensor correlation modeling to maintain tractability, which can degrade the precision of central fusion results. Fusing heterogeneous state representations such as combining pseudo-tracks and full-state tracks, introduces additional complexity. Moreover, residual misassociation errors can occur when closely spaced targets generate ambiguous measurements, even after local filtering. Despite these challenges, the LS-IPDA remains a powerful solution for distributed, communication-efficient, and adaptive multi-target tracking across diverse sensor networks.

### Linear-Complexity Cumulant Multi-Target Filter

### The Linear-Complexity Cumulant Multi-Target Filter is a modern RFS-based algorithm designed to extend the capabilities of traditional multi-target filters like the PHD and CPHD filters while maintaining linear computational complexity. Unlike the PHD filter, which only propagates the first-order statistical moment (the intensity function), the cumulant filter employs factorial cumulants that provides a richer set of statistical descriptors that encode both first- and second-order information about the multi-target distribution. The first-order cumulant corresponds to the standard PHD, representing the expected target density, while the second-order cumulant captures correlations and spatial dependencies between targets. This allows the filter to more accurately model complex target interactions and measurement uncertainty. The approach is built upon the Panjer point process, a generalization of the Poisson process that enables flexible modeling of target births, deaths, and measurement randomness. The algorithm follows a recursive prediction–update structure similar to Bayesian filters: in the prediction step, the cumulants are propagated using survival and motion models; in the update step, new measurements refine the cumulant estimates. Crucially, the filter’s approximations ensure that its computational cost grows linearly with the number of targets and measurements, achieving scalability without sacrificing robustness.

### By leveraging higher-order statistics, the cumulant filter achieves greater robustness and improved cardinality estimation compared to the PHD filter, especially in environments with variable detection probabilities or fluctuating false alarm rates. The inclusion of second-order cumulants provides resilience against noise and clutter, capturing dependencies that simpler moment-based filters ignore. Moreover, the filter retains the linear computational efficiency characteristic of the PHD filter, avoiding the exponential or cubic complexity associated with CPHD, JPDA, or MHT approaches. Like other RFS-based methods, it also eliminates explicit data association, further simplifying computation. However, the cumulant filter inherits some of the classical limitations of RFS-based filters. It does not maintain individual target identities, representing only the collective target distribution. Its performance depends on the accuracy of the approximations used to maintain linearity, and truncating higher-order cumulants can cause information loss in highly nonlinear or strongly coupled systems. Finally, while the second-order cumulant enhances accuracy, it still cannot capture all dependencies in complex multi-target interactions. Nevertheless, the linear-complexity cumulant filter stands as a compelling advancement, offering a practical balance between computational tractability and statistical richness for robust multi-target tracking.

## Comparative Overview of Target Tracking Methods

### This section provides a comparative overview of a broad spectrum of target tracking filters, ranging from foundational model-based estimators such as the Kalman Filter family to advanced Random Finite Set (RFS) and machine-learning-augmented frameworks. The comparison highlights their operating principles, advantages, limitations, and the evolving role of the Kalman Filter (KF) as the field transitions from deterministic models to probabilistic and data-driven paradigms.

### Overview of Tracking Families

### Kalman-based Filters (KF, EKF, UKF). The Kalman Filter (KF) and its nonlinear extensions such as the Extended Kalman Filter (EKF) and Unscented Kalman Filter (UKF), form the foundation of state estimation in tracking. They provide optimal or near-optimal estimation under linear-Gaussian assumptions and remain the backbone for many hybrid tracking systems. While the KF excels in computational efficiency and analytic tractability, its assumptions limit its performance in nonlinear or high-clutter environments. The EKF mitigates this through local linearization, and the UKF further refines this process using deterministic sampling (“sigma points”) to handle stronger nonlinearities without explicit Jacobians.

### Multi-Target Filters (PHD, LMB, Cumulant). As tracking shifted toward multi-object scenarios, Random Finite Set (RFS) theory introduced filters such as the Probability Hypothesis Density (PHD), Gaussian Mixture PHD (GM-PHD), and Labelled Multi-Bernoulli (LMB) filters. These algorithms bypass explicit data association by estimating the collective multi-target state, efficiently handling births, deaths, and clutter. The Linear-Complexity Cumulant Filter extends the RFS framework by propagating higher-order factorial cumulants, enabling more robust cardinality and spatial correlation estimation at linear computational cost. While these probabilistic filters offer scalability and robustness, they often lose explicit track identity and rely on Gaussian or independence assumptions for tractability.

### Data Association and Distributed Filters (IPDA, LS-IPDA). The Integrated Probabilistic Data Association (IPDA) and its distributed variant, Local Sensor IPDA (LS-IPDA), explicitly model measurement-to-track associations, managing individual tracks through the Probability of Target Existence (PTE). LS-IPDA introduces a decentralized architecture, allowing each sensor node to process local measurements and transmit only refined track information to a fusion center. This reduces communication load, enhances scalability, and enables early clutter rejection. However, neglecting inter-sensor correlations and the complexity of state fusion remain challenges in such decentralized systems.

### Machine Learning-Enhanced and Deep Learning Filters (IPDA-MLF, GNN, Transformer).  Recent advances integrate machine learning (ML) and deep neural architectures into traditional filtering frameworks to overcome the limitations of model-based approaches. The Integrated PDA with ML Feedback (IPDA-MLF) leverages learned features such as appearance or contextual similarity, to refine association probabilities, while maintaining a Kalman-like prediction-correction backbone. Graph Neural Networks (GNNs) and Transformers introduce relational and attention mechanisms that model inter-object dependencies and global temporal context. Transformer-based trackers, such as TrackFormer, unify detection and association into a single end-to-end framework, effectively replacing the need for handcrafted motion or measurement models.

| Method | Tracking Scope | Core Principle | Advantages | Limitations |
| --- | --- | --- | --- | --- |
| KF | Single target | Linear-Gaussian estimation | Optimal for linear systems, low cost | Fails in nonlinear or cluttered scenes |
| EKF | Single target | First-order linearization | Handles mild nonlinearity | Linearization errors, instability |
| UKF | Single target | Sigma-point nonlinear propagation | No Jacobians, higher accuracy | More computationally expensive |
| PHD | Multi-target | RFS intensity propagation | No explicit association, robust to clutter | No track identity, poor cardinality accuracy |
| GM-PHD | Multi-target | Gaussian mixture representation | Closed-form, efficient | Gaussian assumption, identity loss |
| SMC-PHD | Multi-target | Particle representation | Handles nonlinearities | Computationally intensive |
| LMB | Multi-target (labelled) | Labelled RFS tracking | Maintains identity continuity | Assumes independence, less optimal than GLMB |
| Cumulant Filter | Multi-target (statistical) | Propagates factorial cumulants | Linear complexity, robust, accurate cardinality | No identity, approximation errors |
| IPDA-MLF | Multi-target (hybrid) | ML-augmented association | Learns contextual cues, robust to clutter | Data-hungry, complex integration |
| LS-IPDA | Multi-target (distributed) | Local IPDA per sensor node | Scalable, low communication, early clutter rejection | Sensor correlation ignored, fusion complexity |
| GNN | Multi-target (context-aware) | Graph-based association | Models inter-object relations | High computation, risk of over-smoothing |
| Transformer MOT | Multi-target (end-to-end) | Attention-based joint detection and tracking | End-to-end, global context | High data/computation demand |

### Role of the Kalman Filter in Multi-Target Tracking

### While originally designed for single-target estimation, the Kalman Filter remains an indispensable building block within modern multi-target tracking (MTT) architectures. In tracking-by-detection systems, a separate KF (or variant) is assigned to each target to perform prediction and noise filtering, while higher-level algorithms handle data association between detections and tracks.

### In RFS-based frameworks like GM-PHD and LMB, the KF is used internally to propagate and update Gaussian components or labelled tracks. In ML-augmented approaches such as IPDA-MLF or Transformer-based MOT, the KF structure is often retained but enhanced by neural components that learn association likelihoods or adapt filter gains dynamically. Even in GNN-based systems, KF-like motion models provide priors for graph reasoning. However, in fully end-to-end deep trackers, the Kalman Filter can be completely replaced by learned dynamical models such as attention-based motion propagation in Transformers, eliminating the need for explicit filtering equations.

### Conclusion

### In summary, target tracking has evolved from deterministic, model-driven filters (e.g., KF, EKF, UKF) to probabilistic RFS-based approaches (e.g., PHD, LMB, Cumulant) and finally to data-driven learning-based frameworks (e.g., GNNs, Transformers). Each generation addresses the limitations of the previous one, moving from precise but brittle models to adaptive and scalable architectures. Despite this evolution, the Kalman Filter remains a conceptual cornerstone, underpinning the mathematical foundation and recursive logic of nearly all modern tracking methodologies.